## Audit Codes Summary

| Code | Filter | Name | Parameters |  |
| --- | --- | --- | --- | --- |
| 00 | OTHER | CORRUPT |  |  |
| 01 | OTHER | POWER_ON | Date, Time, Machine Number, Network Node |  |
| 02 | OTHER | POWER_OFF | Date, Time, Machine Number, Network Node |  |
| 03 | OTHER | BATTERY_LOW_WARNING | Date, Time, Volts |  |
| 04 | OTHER | BATTERY_FLAT_SHUTDOWN | Date, Time, Volts |  |
| 05 | OTHER | BATTERY_LITHIUM_FAIL | Date, Time |  |
| 06 | OTHER/CHARGE | BATTERY_CHARGE_START | Date, Time, Volts, Temperature, Trickle |  |
| 07 | CHARGE | BATTERY_CHARGE_DATA | (technical engineering data - please ignore) |  |
| 08 | OTHER/CHARGE | BATTERY_CHARGE_END | Date, Time, Volts, Temperature, Result |  |
| 10 | SHIFT | SHIFT_START | Date, Time, Shift Number, Portion Number, User Number |  |
| 11 | SHIFT | SHIFT_TEMP_LOGOUT | Date, Time, Shift Number, Portion Number, User Number |  |
| 12 | SHIFT | SHIFT_CONTINUE | Date, Time, Shift Number, Portion Number, User Number |  |
| 13 | SHIFT | SHIFT_TRANSFER_CASHUP | Date, Time, Shift Number, Portion Number, User Number, Shift Total |  |
| 14 | SHIFT | SHIFT_TRANSFER_ABORT | Date, Time, Shift Number, Portion Number, User Number, Shift Total |  |
| 15 | SHIFT | SHIFT_END_CASHUP | Date, Time, Shift Number, Portion Number, User Number, Shift Total |  |
| 16 | SHIFT | SHIFT_END_ABORT | Date, Time, Shift Number, Portion Number, User Number, Shift Total |  |
| 17 | SHIFT | SHIFT_LOGIN_FAILURE | Date, Time, Shift Number, User Number, Reason |  |
| 18 | SHIFT | CUMULATION_START | Date, Time, Type |  |
| 19 | SHIFT | CONFIG_ID | Snapshot Number, Revision Number |  |
| 20 | SHIFT | MERCHANT | Merchant ID, Merchant Q-Code, Terminal ID |  |
| 25 | SHIFT | DECLARE_PAYMENT | Date, Time, Payment Type, Declared Value |  |
| 26 | SHIFT | DECLARE_CASH_INITIAL | Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value |  |
| 27 | SHIFT | DECLARE_CASH_TOPUP | Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value |  |
| 28 | SHIFT | DECLARE_CASH_BANKED | Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value |  |
| 29 | SHIFT | DECLARE_CASH_FINAL | Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value |  |
| 30 | OTHER | EVENT/SAILING_START | Date, Time, Event Date, Event Time, Arrive, Onboard |  |
| 31 | OTHER | EVENT/SAILING_REPORT | Date, Time, Ticketed, Unticketed |  |
| 32 | OTHER | EVENT/SAILING_END | Date, Time, Depart, Onboard |  |
| 33 | OTHER | SET_ROUTE | Date, Time, User, Route, Master Route |  |
| 34 | OTHER | MANUAL_RECEIPT | Date, Time, User, Transaction Number |  |
| 40 | TRX | TRANSACTION | Date, Time, Transaction Number, Transaction Total, Payment Type, Flags |  |
| 41 | TRX | PAYMENT | Payment Type, Currency Type, {Format}, Currency Tendered, Base Tendered |  |
| 42 | TRX | TRX_EXTRA_INFO | Text |  |
| 43 | TRX | CHANGE | Payment Type, Currency Type, {Format}, Currency Change, Base Change |  |
| 45 | TRX | PAYMENT_CHEQUE | Cheque Number, Sort Code, Account Number |  |
| 46 | TRX | PAYMENT_CREDIT_CARD | Card Number, Expire Date |  |
| 47 | TRX | PAYMENT_CREDIT_CARD2 | Start Date, Issue Number, Swiped, CPC |  |
| 48 | TRX | PAYMENT_CREDIT_ACCOUNT | Account Number, Card Number, Order Number |  |
| 49 | TRX | PAYMENT_VOUCHER | Voucher Identifier |  |
| 50 | TKT | TICKET | Type, Bodies, Vehicle, Ticket Number, Route, Value Inc, [Void] |  |
| 51 | TKT | VEHICLE_INFO | Vehicle Length |  |
| 52 | TKT | RETAIL | Type, Value Inc, [Void] |  |
| 53 | TKT | RETURN_TICKET | Type, Bodies, Vehicle, Ticket Number, Route, [Void] |  |
| 54 | TKT | RETURN_VOUCHER | Type, Bodies, Vehicle, Ticket Number, Route, [Void] |  |
| 55 | TKT | VOUCHER_ACCEPTED | Date, Time, Node, Type, Voucher Number |  |
| 56 | TKT | VOUCHER_TICKET | Type, Bodies, Vehicle, Ticket Number, Route |  |
| 57 | TKT | TKT_EXTRA_INFO | Field, Text |  |
| 58 | TKT | VAT | VAT Type, Value VAT |  |
| 59 | TKT | PRINT FAILED | Ticket Number |  |
| 60 | TKT | COMBO | Type, Bodies, Vehicles, Route, Value Inc, [Void] |  |
| 61 | TKT | COMBO_TICKET | Type, Bodies, Vehicle, Ticket Number, [Void] |  |
| 62 | TKT | COMBO_RETAIL | Type, [Void] |  |
| 65 | TKT | MACRO | Type, Bodies, Vehicles, [Void] |  |
| 66 | TKT | MACRO_TICKET | Type, Bodies, Vehicle, Ticket Number, Route, Value Inc, [Void] |  |
| 67 | TKT | MACRO_RETAIL | Type, Value Inc, [Void] |  |
| 70 | VOID | VOID_TRANSACTION | Date, Time, Transaction Number |  |
| 71 | VOID | VOID_ITEM | Date, Time, Transaction Number, Type, Ticket Number |  |
| 72 | VOID | VOID_PAYMENT | Payment Type, Currency Type, {Format}, Currency Voided, Base Voided |  |
| 73 | VOID | VOID_MISC | Date, Time, Payment Type, Value Inc |  |
| 80 | OTHER | SYSTEM_USER | Date, Time, User, Action |  |
| 81 | OTHER | AUDITROLL | Date, Time, User |  |
| 82 | OTHER | SPOTCHECK | Date, Time, User |  |
| 83 | OTHER | CHANGE_DATE | Date, Time, User, New Date |  |
| 84 | OTHER | CHANGE_TIME | Date, Time, User, New Time |  |
| 85 | OTHER | HISTORICAL_CASHUP | Date, Time, User |  |
| 86 | OTHER | CUMULATIVE_CASHUP | Date, Time, User |  |
| 88 | OTHER | DATA_TRANSFER | Date, Time, Type, Result, Reason |  |
| 90 | OTHER | AUDIT_CLEARDOWN | Date, Time, User |  |
| 91 | OTHER | SHIFT_NUMBER | Date, Time, User, Shift Number |  |
| 92 | OTHER | TRANSACTION_NUMBER | Date, Time, User, Transaction Number |  |
| 93 | OTHER | TICKET_NUMBER | Date, Time, User, Ticket Number |  |
| 99 | OTHER | ERROR | Date, Time, Error Number |  |

Note: parameters in [ ] brackets only show up in the uploaded transaction files, and parameters in { } brackets only show up in the audit roll.

Audit Code Descriptions

The following are detailed descriptions of each of the audit codes. Unless explained otherwise, you can assume that any Date/Time fields are the date/time of the event which lead to the audit code being recorded and any User field is the currently logged in user performing the event.

### Code 00

00	CORRUPT

Any audit record will appear in this form if the record has become corrupted, due to a software or hardware fault. Records contain a CRC for specifically detecting when this has occurred. The details contained within the record are not shown as it would not be possible to determine which (if any) fields were actually correct or corrupt.

### Codes 01, 02

01	POWER_ON	Date, Time, Machine Number, Network Node

02	POWER_OFF	Date, Time, Machine Number, Network Node

- Machine Number. The serial number of the ticket machine.

- Network Node. The network/tracs node number of the ticket machine.

These appear at power on and power off and provide identification of the machine. Note, the power on code is only issued once the machine reaches the login prompt, and will not appear if the user diverts off into any fault handling or backdoor administration functions.

### Codes 03, 04

03	BATTERY_LOW_WARNING	Date, Time, Volts

04	BATTERY_FLAT_SHUTDOWN	Date, Time, Volts

- Volts. The actual battery voltage reading which lead to this audit code being issued.

Code 03 is issued when the operator is given a battery low warning.

Code 04 is issued when the machine decides to shutdown because battery power is so low. The code may not be issued if power is so critical that control does not return to the main idle screen (which is where the shutdown is activated).

### Code 05

05	BATTERY_LITHIUM_FAIL	Date, Time

Issued at bootup if the machine detects a failure in the lithium backup battery.

### Codes 06, 07, 08

06	BATTERY_CHARGE_START	Date, Time, Volts, Temperature, Trickle

07	BATTERY_CHARGE_DATA	(technical engineering data - please ignore)

08	BATTERY_CHARGE_END	Date, Time, Volts, Temperature, Result

- Date/Time. The date/time that the fast charge started/ended (excluding any trickle period).

- Volts. The battery voltage (volts) at fast charge start/end.

- Temperature. The battery temperature (degrees centigrade) at fast charge start/end.

- Trickle. The number of minutes spent on pre-charge trickle.

- Result. The outcome of the fast charge.

1 - Completed successfully.

2 - Aborted by user.

3 - Failed, Too Cold.

4 - Failed, Too Hot.

5 - Failed, Time Limit.

6 - Failed, +DeltaT.

7 - Failed, -DeltaV.

Issued during a Fast Charge. A code 06 will always be issued whenever fast charging begins, and a code 08 when it completes, regardless of the outcome.

A series of one or more code 07s will occur between the code 06 and code 08 containing technical parameters about the charge. These code 07s have their own audit roll filter category and are disabled by default, so you'll only see them if you enable the filter or look at the uploaded transaction files.

### Codes 10, 11, 12, 13, 14, 15, 16, 17

10	SHIFT_START	Date, Time, Shift Number, Portion Number, User Number

11	SHIFT_TEMP_LOGOUT	Date, Time, Shift Number, Portion Number, User Number

12	SHIFT_CONTINUE	Date, Time, Shift Number, Portion Number, User Number

13	SHIFT_TRANSFER_CASHUP	Date, Time, Shift Number, Portion Number, User Number, Shift Total

14	SHIFT_TRANSFER_ABORT	Date, Time, Shift Number, Portion Number, User Number, Shift Total

15	SHIFT_END_CASHUP	Date, Time, Shift Number, Portion Number, User Number, Shift Total

16	SHIFT_END_ABORT	Date, Time, Shift Number, Portion Number, User Number, Shift Total

17	SHIFT_LOGIN_FAILURE	Date, Time, Shift Number, User Number, Reason

- Date/Time. The date/time of the login/logout.

- Shift Number. The shift number being started/continued/ended.

- Portion Number. The portion number within the shift, if multi-user mode is being used.

- User Number. The user starting/continuing/ending the shift.

- Shift Total. The total financial takings for the shift.

- Reason. See relevant code description below.

Code 10 appears only at the start of a new shift. A shift is everything from the code 10 to the code 15.

Code 11 can appear at any point within a shift as a result of a temporary logout. Normally, it will eventually be followed by a code 12, unless an administrator decides to force an end to the shift without logging back into the shift.

Code 12 continues a shift after a previous temporary logout or transfer cashup. If after a temporary logout, the user continuing the shift may be either the same user who previously logged out, or it may be an inspector/manager override. If after a transfer logout, the user could be any user and may continue either an existing portion or create a new portion.

Code 13 appears at the end of a portion within a shift, if that transfer was successful. Normally it will eventually be followed by a code 12, unless an administrator decides to force an end to the shift without logging back into the shift.

Code 14 appears at the attempted end of a portion within a shift, if that transfer was unsuccessful, in which case no logout was performed. The code is provided only to aid in detecting operator fraud.

Code 15 appears only at the end of a shift, if the end shift cashup was successful.

Code 16 appears only at the attempted end of a shift, if the end shift cashup was unsuccessful, in which case no logout was performed. The code is provided only to aid in detecting operator fraud.

Code 17 appears if there was an error attempting to start/continue a shift. Reason codes are:

- Multi-user limit exceeded (I.e., no space to create a new portion).

- Wrong user (I.e., only the user who performed temporary logout (or inspector/manager) is allowed back in.

- Unrecognised user (I.e., a user not defined in the logon user table). In this case, the user number shown will be zero.

### Code 18

18	CUMULATION_START	Date, Time, Type

- Type. The type of cumulation (0=Daily, 1=Weekly, 2=Monthly, 3=User)

Issued whenever the system starts a new cumulative shift (which normally destroys the oldest cumulative shift of that type).

### Code 19

19	CONFIG_ID	Snapshot Number, Revision Number

- Snapshot Number. Internal 8 digit revision numbed used only by Windows Reporting. This will be all zeros if an unpublished configuration was downloaded, and Reporting will not be able to include this shifts data in any reports.

- Revision Number. The user visible revision number for this configuration.

This code is added once at the start of a shift to provide, primarily for Windows Reporting, information which can be used to identify which configuration was active during this shift.

### Code 20

20	MERCHANT	Merchant ID, Merchant Q-Code, Terminal ID

- Merchant ID. The merchant ID used throughout this shift (possibly dependent on Route).

- Merchant Q-Code. The merchant Q-Code used throughout this shift (possibly dependent on Route).

- Terminal ID. The merchant terminal ID.

This code is issued during the shift start sequence to identify various parameters that are needed for subsequent credit card processing with an appropriate payment service provider.

### Code 25

25	DECLARE_PAYMENT	Date, Time, Payment Type, Declared Value

- Payment Type. The payment type being declared.

- Declared Value. The value entered.

This code appears at the transfer/end of a shift/portion for any payment type that has declarations enabled, but since a cashup report may be later aborted, there may be multiple code 25s for a given portion - do not tally these - the last code 25 is the last known amount declared.

### Codes 26, 27, 28, 29

26	DECLARE_CASH_INITIAL	Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value

27	DECLARE_CASH_TOPUP	Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value

28	DECLARE_CASH_BANKED	Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value

29	DECLARE_CASH_FINAL	Date, Time, Shift Number, Portion Number, Currency Type, {Format}, Value

- Shift Number. The shift number the cash declaration relates to.

- Portion Number. The portion number within the shift, if multi-user mode is being used.

- Currency Type. The number of the currency used (from 0 upwards, 0 is default/base currency).

- Format. This special byte presently appears only in the audit roll, and not in the uploaded transaction files, and is simply used to allow the transaction upload files to properly format foreign currencies. May be removed in future.

- Value. The amount declared, in this currency

These cash declaration codes track the cash float which may be shown in the Currencies section on a cashup report. Code 26 will only appear if the feature is enabled and the user doesn't cancel the input. It will appear only once for any given portion, which means it may appear at the start of a shift and at the start of any new portions after a shift transfer. Codes 27 and 28 appear as often as these functions are performed, and the total cash topup/banked can be found by tallying each occurrence. Code 29 will only appear if the feature is enabled and the user doesn't cancel the input. It will appear whenever an attempt is made to transfer/end a shift/portion, but since a cashup report may be later aborted, there may be multiple code 29s for a given portion - do not tally these - the last code 29 is the last known amount declared.

### Codes 30, 31, 32

30	EVENT/SAILING_START	Date, Time, Event Date, Event Time, Arrive, Onboard

31	EVENT/SAILING_REPORT	Date, Time, Ticketed, Unticketed

32	EVENT/SAILING_END	Date, Time, Depart, Onboard

- Event Date/Time. The actual date/time of the event, which may or may not be the same as the date/time that the event was started and for which a code 30 was issued.

- Arrive. The number of people arriving, as declared by the Operator.

- Onboard. The number of people onboard (after arrive/depart taken into account).

- Ticketed. The number of people ticketed in this event/sailing.

- Unticketed. The number of people who still need to be ticketed.

- Depart. The number of people departing, as declared by the Operator.

Code 30 is issued at the start of an event/sailing. Code 32 is issued at the end of an event/sailing. Code 31 is issued whenever a manual event/sailing report is requested, or automatically alongside a code 30/31 if the relevant option is enabled to force a report at sailing start/end.

All ticket/transaction items occurring between a code 30 and a code 32 are logged against that event/sailing. Any ticket/transaction items occurring outwith a code 30/32 pair are not considered to be sold against any event/sailing.

Note: there may not be an EVENT/SAILING_END code issued if an End Shift takes place during an open event/sailing. This is just an assumed end of event/sailing also.

### Code 33

33	SET_ROUTE	Date, Time, User, Route, Master Route

- Route. The new route number from 1 to 250.

- Master Route. The existing master route number from 0 to 250 (as previously defined in Local Settings).

Issued whenever the user changes the default route. Also issued at shift start regardless of whether the user selected a route or not. This code is not issued when a Hotkey makes a temporary route change. Instead, all ticketing audit codes specify which route was in operation for that ticket. All other audit codes can be assumed to be performed against the current route, which is the last route number seen from a code 33.

### Code 34

34	MANUAL_RECEIPT	Date, Time, User, Transaction Number

- Transaction Number. The transaction number that the receipt relates to.

Issued whenever a user manually requests a transaction receipt (automatic receipts are not recorded since the assumption is they were issued).

### Codes 40, 41, 42, 45, 46, 47, 48, 49

40	TRANSACTION	Date, Time, Transaction Number, Transaction Total, Payment Type, Flags

41	PAYMENT	Payment Type, Currency Type, {Format}, Currency Tendered, Base Tendered

42	TRX_EXTRA_INFO	Text

43	CHANGE	Payment Type, Currency Type, {Format}, Currency Change, Base Change

45	PAYMENT_CHEQUE	Cheque Number, Sort Code, Account Number

46	PAYMENT_CREDIT_CARD	Card Number, Expire Date

47	PAYMENT_CREDIT_CARD2	Start Date, Issue Number, Swiped, CPC

48	PAYMENT_CREDIT_ACCOUNT	Account Number, Card Number, Order Number

49	PAYMENT_VOUCHER	Voucher Identifier

These appear during a transaction sale, after all of the code 50s/60s for the items being issued. Code 40 always appears but the other codes may appear when appropriate as explained below.

For Code 40:

- Date/Time. The date/time for the whole transaction (every ticket in a transaction always have the same date/time).

- Transaction Number. The transaction sequence number from 000001 to 999999.

- Transaction Total. The total value of the transaction (in default currency).

- Payment Type. If the transaction was paid for with a single payment (the normal), this will be the number of that single payment type (from 1 upwards). If there were multiple payments, this will be zero.

- Flags. Miscellaneous payment flags byte generated by adding any or all of the following values:

1 - Customer receipt has been printed.

2 - Seller receipt has been printed.

4 - Customer sales voucher has been printed.

8 - Seller sales voucher has been printed.

128 - Transaction has been stealthed - financial data in the transaction will not have been included in the cashup.

- For Code 41:

- Payment Type. The number of the payment type that this payment represents (from 1 upwards).

- Currency Type. The number of the currency used (from 0 upwards, 0 is default/base currency).

- Format. This special byte presently appears only in the audit roll, and not in the uploaded transaction files, and is simply used to allow the transaction upload files to properly format foreign currencies. May be removed in future.

- Currency Tendered. The amount tendered, in the tender currency.

- Base Tendered. The amount tendered, in base currency.

For Code 42:

- Text. This is the optional extra info entered against the transaction. The code 42 will be present if the system asked the user to enter text, but may be blank if nothing was entered.

For Code 43:

- Currency Change. The amount of change given, in change currency.

- Base Change. The amount of change given, in base currency.

For Code 47:

- Start Date. This will be the start date of the card, or zero if the card does not have a start date.

- Issue Number. This will be the issue number of the card, or -1 if the card does not have an issue number.

- Swipe. This will be 1 if the card was swiped, or 0 if it was keyed.

- CPC. This will be 1 if the card is a Corporate Purchase Card, 0 otherwise.

For Code 46:

- Card Number. As of v1.11, this credit card number is now encrypted.

For Code 48:

- Account Number. The customer's account number. Always present.

- Card Number. The customer's card number. Zero if no card number exists. (This field only appears from V1.20 onwards).

- Order Number. This is an optional field that may be blank if no order number was input.

For Code 49:

- Voucher Number. This is an optional field that may be blank if no number was input.

For transactions with a single payment, the code 41 will only follow the code 40 if the transaction was tendered (an amount of money was keyed in), for which there may also be change given. If no tendering was involved, the full value of the transaction from the code 40 is assumed to have been paid into this payment type and no code 41 will appear.

For transactions with multiple methods of payment, the code 40 will be followed with a code 41 for each payment made, showing how much was taken against each payment type.

In either case, the 40 will first be followed by a 42 if the system has enabled the input of a transaction user text field. If any change exists it will follow everything else. The system currently only supports a single change record which will be in base currency.

For Codes 45-49:

If a non-cash payment is used in the transaction, there will be one of these codes given for that payment, showing the additional payment type specific information relating to that payment. They will always appear after the code 41 to which they pertain, unless the transaction is single payment only without a code 41, in which case this code will appear after the code 40. Both credit card entries (46/47) always appear in pairs.

In summary, the possible sequence of codes seen will be:

For single method of payment:

| Mandatory | Code 40 | (with Payment Type set to non-zero) |
| --- | --- | --- |
| Optional | Code 42 | (only if user text field enabled) |
| Optional | Code 41 | (only if tendered) |
| Optional | Code 45, 46/47, 48 or 49 | (only if non cash) |
| Optional | Code 43 | (only if change) |

For multiple methods of payment:

| Mandatory | Code 40 | (with Payment Type set to zero) |
| --- | --- | --- |
| Optional | Code 42 | (only if user text field enabled) |
| Then, for each payment: |  |  |
| Mandatory | Code 41 | (only first occurrence has valid change value) |
| Optional | Code 45, 46/47, 48 or 49 | (only if non cash) |
| Optional | Code 43 | (only if change) |

Since many customers only ever use single currency Cash without tendering, most transactions are likely to be single payment Cash transactions consisting only of a code 40, thus making most efficient use of audit memory.

### Code 50

50	TICKET	Type, Bodies, Vehicle, Ticket Number, Route,  Value Inc, [Void]

- Type. List Tickets are represented by a number between 1 and 999. Matrix Tickets are represented by a four digit number, two for the Type, followed by two for the Description, so "0206" would be Type 2 Description 6.

- Bodies. The number of bodies on this ticket.

- Vehicle. Either "1" or "0" indicating whether there is a vehicle counted for this ticket.

- Ticket Number. The ticket number from 000001 to 999999.

- Route. The route number the ticket was issued on.

- Value Inc. The ticket value, including VAT if appropriate.

- Void. Set to 1 if item was voided. Only shows in transaction files, not in the audit roll.

This code appears each time a normal List or Matrix ticket is sold in a transaction. Combos, Macros & Retails have their own codes.

### Code 51

51	VEHICLE_INFO	Vehicle Length

This is issued after any ticket code (50,61,66) which represents a vehicle for which a length was input by the operator.

### Code 52

52	RETAIL	Type,  Value Inc, [Void]

- Type. A Retail Item index from 1 to 999. Not the PLU code.

- Value Inc. The item value, including VAT if appropriate.

- Void. See definition on code 50.

This code is issued each time a Retail Item is issued, except during Combos/Macros which have their own codes for Retail Items.

### Codes 53, 54

53	RETURN_TICKET	Type, Bodies, Vehicle, Ticket Number, Route, [Void]

54	RETURN_VOUCHER	Type, Bodies, Vehicle, Ticket Number, Route, [Void]

- All field meanings as per code 50.

Code 53 is issued for the 2nd portion of a Return Ticket, and code 54 for the 2nd portion of a Return Voucher. The 53/54 will be issued after the ticket (1st portion) to which they relate (50,61,66). Both portions are inseparable in that they share one overall price which is given in the 1st portion audit code, and may only be voided together. Accordingly, any additional ticket records such as VAT (58) and VEHICLE_INFO (51) will be given after the 1st portion audit code. The Ticket Number on the 2nd portion will be the one following that in the first. All other information on the 2nd portion audit code is merely a copy of that from the first for convenience.

### Code 55

55	VOUCHER_ACCEPTED	Date, Time, Node, Type, Voucher Number

- Date/Time. The date/time that this voucher is scanned.

- Node. The node number of the machine that issued the voucher.

- Type. The generic ticket type to issue, from 1 to 250, or 0 for "default/unknown".

- Voucher Number. The ticket/voucher number of the voucher, as generated by the issuing machine.

Issued when a voucher is scanned and accepted. Followed by a code 56.

### Code 56

56	VOUCHER_TICKET	Type, Bodies, Vehicle, Ticket Number, Route

- Type. The generic ticket type actually issued.

- Bodies. The number of bodies on the generic ticket.

- Vehicle. Either "1" or "0" indicating whether there is a vehicle counted for this ticket.

- Ticket Number. The ticket number issued to the generic ticket, different from the voucher number.

- Route. The route number the ticket was issued on.

Issued after a code 55 to give details of the generic ticket that was printed. Voucher scanning never occurs during a transaction, so codes 55 & 56 always occur outwith the normal code 40/50 style sequences. The 56 may be followed by a code 59 if the generic ticket failed to print successfully.

### Code 57

57	TKT_EXTRA_INFO	Field, Text

- Field. The field number of this extra info record, from 1 to 4.

- Text. This is the extra info text entered by the user.

This is issued after any ticket code (50,60,65) if the ticket requested an extra info text input and the user keyed something in.

As of v1.16, there are now 4 extra info fields available, and the "Field" code above tells you which one this record applies to. The data input may now also be longer than the audit record has space for, so the first 13 characters will appear in the first record, and the remaining characters on a second record. For example, the following inputs will generate the following codes:

Ticket Extra Info 1: DIRECTIONAL DATA SYSTEMS

Ticket Extra Info 2: 4243

57 1 DIRECTIONAL D

57 1 ATA SYSTEMS

57 2 4243

### Code 58

58	VAT	VAT Type, Value VAT

- VAT Type. Zero is the exempt rate (which do not get recorded), so this will be from one upwards.

- Value VAT. This is the actual amount of VAT calculated.

This is issued for any ticket/retail item which contains VAT. It will be issued after the associated ticket record (the one containing the financial value of the ticket). The fields are:

### Code 59

59	PRINT FAILED	Ticket Number

- Ticket Number. The number of the ticket which failed to print.

This is issued whenever a printed ticket fails to print due to a paper out condition which the user did not rectify. The code always appears after the 50,53,54,56,61,66 to which it applies.

### Codes 60, 61, 62

60	COMBO	Type, Bodies, Vehicles, Route, Value Inc, [Void]

61	COMBO_TICKET	Type, Bodies, Vehicle, Ticket Number, [Void]

62	COMBO_RETAIL	Type, [Void]

- All field meanings as per code 50.

A code 60 is issued when a Combo List Ticket is about to be printed. As each subitem prints, they are recorded with the relevant type of subitem code 61/62. As for the normal ticket code 50, the 61 may be preceded by a 59 if the subitem failed to print or succeeded by a 51 for tickets with vehicle lengths.

Combo items are priced & VATed according to the definition of the Combo itself, so all relevant financial information belongs with the code 60. The 61/62 simply show the details of what the subitems were.

### Codes 65, 66, 67

65	MACRO	Type, Bodies, Vehicles, [Void]

66	MACRO_TICKET	Type, Bodies, Vehicle, Ticket Number, Route, Value Inc, [Void]

67	MACRO_RETAIL	Type, Value Inc, [Void]

- All field meanings as per code 50.

A code 65 is issued when a Macro List Ticket is about to be printed. As each subitem prints, they are recorded with the relevant type of subitem code 66/67. As for the normal ticket code 50, the 66 may be preceded by a 59 if the subitem failed to print or succeeded by a 51 for tickets with vehicle lengths.

Macro items are priced & VATed according to each item issued, so the code 65 simply shows which Macro was issued (along with total bodies/vehicles for convenience), the financial information belongs to each individual subitem issued.

### Codes 70, 71, 72

70	VOID_TRANSACTION	Date, Time, Transaction Number

71	VOID_ITEM	Date, Time, Transaction Number, Type, Ticket Number

72	VOID_PAYMENT	Payment Type, Currency Type, {Format}, Currency Voided, Base Voided

(These codes used to be located at 20,21,22.)

- Transaction Number. The transaction to which the void pertains.

- Type. The ticket type, as per code 50.

- Ticket Number. The ticket number, or zero if item does not have a ticket number.

- Payment Type. The payment type that the refund is deducted from.

- Currency Type. The currency that the refund is being performed in.

- Format. This special byte presently appears only in the audit roll, and not in the uploaded transaction files, and is simply used to allow the transaction upload files to properly format foreign currencies. May be removed in future.

- Currency Voided. The amount voided, in the specified currency type.

- Base Voided. The amount voided, in base currency.

These codes are issued whenever transactions or items within transactions are voided. If a request is made to void the whole transaction, code 70 will appear first. A code 71 will then appear for each item you attempt to void. Finally, a code 72 will exist for each payment that is being voided.

As well as recording these audit codes when the void procedure is performed, the system will also go back and mark the original sale records of the affected items as being voided, and this will show up in the [void] parameter in the transaction files for these records. This allows us to detect attempts to void the same item twice, in which case the void will have a code 70/71 as appropriate, but the code 72 will not include the value of the item, since it has already been deducted somewhere else. The code 70/71 are provided for security/convenience only.

### Code 73

23	VOID_MISC	Date, Time, Payment Type, Value Inc

(This code used to be located at 23.)

- Payment Type. The payment type the deduction was made against.

- Value Inc. The amount, including VAT, of the deduction.

This is a standalone void code used for the "Miscellaneous Void" command which simply allows miscellaneous amounts of money to be voided, unrelated to any ticket/transaction.

### Code 80

80	SYSTEM_USER	Date, Time, User, Action

- Action. Defines what action is being recorded, which is one of the following:

- System user has logged out of the system menu.

- System user has logged in to the system menu.

- System user has been allowed straight in to the system menu.

- System user has exited the system menu continuing as the system user.

Issued whenever any type of login or logout occurs on the System menu.

### Code 81

81	AUDITROLL	Date, Time, User

Issued whenever an audit roll is printed.

### Code 82

82	SPOTCHECK	Date, Time, User

Issued whenever a spotcheck is printed.

### Codes 83, 84

83	CHANGE_DATE	Date, Time, User, New Date

84	CHANGE_TIME	Date, Time, User, New Time

- Date/Time. The date/time of the change.

- New Date. The new date.

- New Time. The new time.

Issued when a system user changes the date (code 83) or the time (code 84). The fields are:

### Codes 85, 86

85	HISTORICAL_CASHUP	Date, Time, User

86	CUMULATIVE_CASHUP	Date, Time, User

Issued whenever a Historical Cashup (code 85) or a Cumulative Cashup (code 86) is printed.

### Code 88

88	DATA_TRANSFER	Date, Time, Type, Result, Reason

- Type. Which type of data transfer this is.

(1=Partition Upload, 2=Partition Download, 3=Config Download, 4=Data Upload).

- Result. Set to 1 if transfer successful, else 0 if it failed.

- Reason. If it failed, this is why (0=Unknown, 1=Abort, 2=Error, 3=No New Data, 4=Timeout).

Issued at the end of a Configuration Download or a Transaction Data Upload.

### Code 90

90	AUDIT_CLEARDOWN	Date, Time, User

Issued whenever the audit memory is cleared (by an administrator).

### Codes 91, 92, 93

91	SHIFT_NUMBER	Date, Time, User, Shift Number

92	TRANSACTION_NUMBER	Date, Time, User, Transaction Number

93	TICKET_NUMBER	Date, Time, User, Ticket Number

Issued whenever a manager manually adjusts the next Shift Number (code 91), the next Transaction Number (code 92) or the next Ticket Number (code 93).

### Code 99

99	ERROR	Date, Time, Error Number

- Error Number. This is an internal software error code (0-255). Current possible errors are:

- Shift Activation Error. The login code failed to locate or create a shift portion for the login user and failed to understand why. The shift header or the data in the shift cache is probably corrupt, so the user is forced to reboot and "fix" the error by nuking the shift in progress (after optionally attempting a cashup).

- Bad Cumulation Index. One of the cumulation cache indexes was detected out of range either just prior to login or as a new cumulation was about to start. The cumulation in question is disabled. Either the cumulation index was corrupt (this will auto-fix next time the cumulation starts), or there is not enough room in the cache for the cumulation (the user should repair the partition settings or cumulation cache division settings).

- Clock Corrupt. The RTC self check has detected that the clock verification pattern that was previously left in the clock's RAM has been modified, suggesting that the clock may have suffered a failure or been overwritten. The test pattern will be reprogrammed to sense this occurring again, but the user should check the clock for the correct date and time.

X.	Any other error codes are unknown errors.

Issued if an abnormal error is detected - not usable by the customer.