General DT30 Operations

Switch On: 	Hold in the black button on the left-hand side for 3 or so seconds.

Scrolling: 		Use the “6” key for scrolling up and the “E” key for scrolling down(or the black knob on side of the unit)

Logging in: 	Enter your ID number and then your PIN, and press OK.

The sample file log in:

Operator: User 101, Pin 101

System Options: At Log In: 000

MGR: User 606, Pin 606

Selling Tickets: 	Hotkey tickets:(select button A, B, C ect + OK)      or

Select the quantity, ticket type (A,B,C), + OK

List Tickets: Press the shift button (the button just left of the OK button) and then enter the List ticket number and press OK > Press OK Once more and it will select the ticket > The Unit then gives you one more opportunity to select another ticket > If no other ticket is desired then press OK once more and it will subtotal and print.

Receipt: 		Fn + RECEIPT + [scroll to transaction number] + Ok + OK + Ok.

Void (Whole Transaction): 		Fn + VOID + [scroll to transaction number] + Ok.

Void (Ticket Within a Transaction): 	Fn + VOID + [scroll to transaction number] + Fn + [scroll to ticket number] + OK.

*****End Shift:

Fn + Exit + [scroll to End Shift Cashup] + OK + OK

* The double-OK before and after the cashup report prints is to allow cancellation of accidental cashups, and re-load of paper for crumpled or poorly printed cashups. Either of these items may be restricted to manager use only.